PROBLEM - I Had 2 1.2mb 5.25 floppy drives installed in

my 386 system and wanted to add a CMS tape

backup system without giving up a floppy. I

could purchase an add on card but it seemed

high priced to me.

SOLUTION - make a cable to switch the "B" drive connector

from the floppy to the tape drive. Anytime I'm

not using the tape drive I leave the switch set

to the floppy drive. When I get ready to backup

I switch to the tape. The cost was less than $10.

DETAILS - There are 2 critical lines that require switching.

Thes are named Motor Enable B and Drive Select B

in the IBM Technical Reference manual. A Double

Pole Double Throw toggle switch switches these

lines between devices.

CONTSTRUCTION -

34 pin connector

on controller                                     "B"     "B"   "A"

-    10 motor enable A                            -       -     -

| ----------------------------------------------| |-----| |---| |

|   12 Drive select B                           | |     | |   | |

| ---------------------------            -------| |  ---| |---| |

|   14 Drive select A        |          |       | | |   | |   | |

| ---------------------------)----------)-------| |-)---| |---| |

|   16 Motor enable B        |          |       | | |   | |   | |

|----------------------      |          |      -| | |  -| |---| |

|                      |     |          |     | | | | | | |   | |

|                      |     |          |     | | | | | | |   | |

-                       |     |    ------      |  -  | |  -     -

|      -->             |     | |

|          ------------)-----  |

|                      |       |

|          ------------        |

------->                      |

--------------------

DPDT Switch

Excuse the art work, but I hope you can figure it out with

this explanation. I made a cable with 3 34 pin disk connectors.

pins 10-16 are twisted between the last two like a standard

IBM type cable. Pins 12 and 16 are cut at a convenient point

after they come off the controller and before each of the 2

"B" drive connectors. take the wire and solder on an extension

and run it to a Double Pole Double Throw Switch. Next run

wires from each side of the switch to the short pieces of

wire coming from the connectors where you previously cut.

Solder the connections and I recommend using heat shrink

tubing for insulation on each connection. Now run the cable

to the Tape drive, to the "B" disk drive and then to the

"A" disk drive. Mount the DPDT switch in a convenient place

and you're set.

I had 3 drive openings in my clone case so I mounted the tape

drive on the bottom and the switch in a small hole drilled in

the front panel next to the keyboard lock switch.

A friend who had only 2 openings mounted the tape inboard

and got a case for the floppy drives and just made the

cable long enough to go external.